SEAWOLF PILOTHOUSE 36

“Discovery”

HULL SPECIFICATIONS:

Length	 		35’11”

Beam	 		11’2”

Deadrise aft	14 degrees

Deadrise fwd	35 degrees

Bottom		.250” 5086 Aluminum

Sides and bulkheads	.190” 5086 Aluminum

Side Height aft	51.5”

Decks			.190” 5052 Aluminum

Cabin sides and bulkheads	            .160” 5052 Aluminum

Cabin roof		            .125” 5052 Aluminum

Weight, dry with twin D6 310            12,110 lbs

Performance, wet, loaded, top speed 41 knots

Cruise @ 2600 RPM, 27 Kts

STANDARD EQUIPMENT:

Propulsion:

2-Volvo Penta D6 310 diesel Duoprop I/O

EVC Electronic Vessel Control

Instrument panels with EVC engine and fuel data, tachometer, hourmeter,

engine temp, oil pressure, fuel level, voltmeter, drive trim gauge

and wiring harness

Fresh water cooling

2-Racor 500 water separating fuel filter

Volvo electronic engine controls three station)

Teleflex hydraulic steering, with engine power assist

Engine compartment sound insulation

SEAWOLF 36 “Discovery”

(STANDARD EQUIPMENT-cont.)

Hull, general:

Cockpit deck, flush, self-draining

Bowrails and cockpit handrails 1” s-40 aliminum pipe, welded

Cabin top handrails, 1” aluminum pipe, welded

Radar arch, 1 1/2”aluminum pipe, welded, with antenna and GPS mounts

Anchor roller, ¼” plate, welded with 3” roller

Anchor rode locker, below deck, with drains and access hatch

Swimstep, 28” welded, with welded transom grab handles

Cleats, seven 10” cast aluminum, welded

Fairlead chocks ,cast aluminum, welded

Engine hatches, two, with gas lifting struts

Storage holds, two, self draining, under cockpit deck 60”L x 30”W x 15” D

Fuel tank, one, 329 gallon, bolt in w/gauge sender

Fuel tank access, bolt down plate on deck to access entire fuel tank

Fuel tank quick access, Bomar C4810 cast aluminum access hatch on deck

Bilge pumps, 3-Jabsco 3700 GPH w/automatic float and panel switches

Trim-tab system, Boat Leveler 12" x 18"

Running lights, Aqua Signal series 20

Horn, AFI 10029 XLP electric dual trumpet

Deck light, Jabsco 4"x 6" halogen mounted on mast

Windlass; Lewmar 1000H electric, with deck mount foot switches

Doors and windows, All heavy duty aluminum frame Diamond /Seaglaze

With ¼” tempered glass

Extensive acoustic insulation throughout

1-Zinc anode on hull

Large comfortable finished forecabin

Quiet, comfortable pilothouse style main cabin

All interior stick built, with structural aluminum welded in place

SEAWOLF 36 “Discovery”

(STANDARD EQUIPMENT-cont.)

Electrical System DC;

Batteries, house, 4-Exide GC 145 6 Volt Deep cycle (490 amp hours)

Battery, engine, one Exide gp 31HD 1025 CCA

Battery switches, BEP heavy duty with emergency parrellel

Automatic charging relay, Blue Sea Sysyems

Main DC circuit breakers, Blue Sea Systems 185 series

Electrical panels, 2-Blue Sea Systems 8023 with 8 toggle circuit breakers each

Dash switch panel, 10 Contura rocker switch with push-to-reset circuit breakers

12V DC outlet, one at dash

Wiring, all fine stranded marine grade duplex tinned copper

Connectors and fittings, all “Ancor” marine grade

Electrical System AC;

Shore power system, 30 Amp,

Power inlet, Marinco 303 ELB

Shore power cord, 50' Marinco

Electrical panel AC, Blue Sea 8027 with main breaker, polarity indicator

and 5 toggle breakers for branch circuits

Electrical outlets, 3- duplex, one in forcabin, one at galley and one at dinette

Shore power Isolation transformer, Charles IXFMR 30

All fine stranded marine grade triplex tinned copper wire

All Ancor marine grade connectors and fittings

Fresh Water System;

Fresh water tank, 68 gallon,aluminum, bolt in

Fresh water pump, Jabsco Sensor Max 4.5 gpm variable speed automatic

Galley sink, Polar stainless, single 9”deep

Galley faucet, Skandvic 10488 single lever

Water heater, Isotherm 11 gallon heat exchanger/ 120 volt

Shower drain system, Attwood 4143-4 automatic

Washdown faucet on deck (fresh water)

SEAWOLF 36 “Discovery”

(STANDARD EQUIPMENT cont.)

Head system:

Toilet, Jabsco Quiet Flush Electric

Holding tank, Sealand 28 gallon heavy duty rotomolded plastic

With vent filter system and tankwatch monitor

All fittings and penetrations on top of tank

Waste discharge pump, Sealand T-12 electric diaphragm

Deck discharge for dockside vacuum system

Sink, Polar stainless, oval with Scandvic combination faucet/shower

Shower drain system

Waterproof self retracting toilet paper holder

Cabin / Interior;

Forecabin 8’4”length:

King berth (6’4” x 6’8”) with 6” upholstered cushion

Hanging locker starboard with one drawer under

Storage shelf port, starboard and forward with carpet liner and teak trim

One storage locker each side with sliding doors, shelf, carpet liner and teak trim

Portlites, 4-Lewmar Ocean series, opening with screens

Skylight hatch, Bomar 1029-10 with screen

Panel headliner with acoustic foam insulation

Cabin lights, 2-red / white

Access to anchor locker

Heavy duty rubber flooring

Marine carpet hull liner

Head compartment portside, painted

Teak entry door with Perko latch with lock

Storage locker with teak louver door

Toilet paper holder, waterproof, automatic retracting

Stainles grab bars

Light, red / white

Main cabin, pilothouse style, 10’8” length;

Helmstation starboard side with teak dash and console

Helmseat, Bentleys Rivermaster with folding armrests on Garelick slide

Storage cabinet, under helmseat with one shelf and door

Storage, open under helm floor

Storage, cabinet below galley sink with one shelf and door

SEAWOLF 36 “Discovery”

(STANDARD EQUIPMENT cont.)

Cabin / Interior cont.

Nova-Kool 7.5 two door refrigerator/ freezer

Space for propane or diesel galley stove

3 storage drawers

Hanging locker

Stove, Wallas 125 DU diesel with pot holders

Dinette portside, includes;

Table, 30”x 46”with teak sea rail

Dinette converts to berth 46”x 73”

Bench seats, face to face with storage under with top access

Dinette cushions 4” with upholstery your choice

Companion seats forward, 2-Bentleys Rivermaster with folding armrests

Storage, under companion seats with top and side access

Storage, under companion floor with top access

Storage, 4 huge 16”x 30” x9” drawers under dinette

Access hatch in dinette floor

Furnace, Webasto AT 3900 diesel with 4 outlets

Defrost, engine heat, 3 outlets ducted to windshield

Cabin heater, engine heat, Heatercraft 400H

Windows, forward, 3 Diamond Seaglaze D5 fixed, with windshield wipers

Windows, side, 2 each side, sliding, Diamond Seaglaze 106S

Windows, rear, 2, Diamond Seaglaze D5 fixed

Entry door, heavy duty aluminum, Diamond Seaglaze, with drop-slide window and

stainless Trio-Ving deadbolt lockset

Windshield wipers, Imtra, Roca WP with AFI stainless pantograph arms

Companion seats, 2-Bentleys Rivermaster, portside forward with folding armrests

Convertible dinette portside aft with drawers under

Headliner, soft touch with acoustic foam insulation and 4 cabin lights, (red/white)

Heavy duty rubber flooring

Teak trim

NOTE;

All interior framing for all cabinetry and components are constructed of aluminum,

welded in place to the hull structure for maximum strength, durability and versatility.

SEAWOLF 36 “Discovery”

(STANDARD EQUIPMENT cont.)

Flying bridge (optional)

Cabin roof reinforcement

Tall fold down arch with Radar, GPS, VHF antenna and spotlight mounts

Flying bridge hand rails and ladder

2-Bentleys Rivermaste or Guide helmseats with fixed arms

Wind deflector for comfortable cruising

Console with hydraulic steering, Volvo EVC controls, EVC tachometers with

engine and fuel data EVC drive trim and trim tab control

Compass, Ritchie ss-2000

Huge storage area under console with 2 Bomar hatch access

Sunbrella covers for seats and instrument console

Tie downs for dinghy